AUTONOMOUS DRONE NAVIGATION

DA-1 Complete Project Material

3 Members • Equal Workload • ROS 2 + PX4 + Gazebo

# 1. Project Idea, Background & Significance

## 1.1 Project Idea

The proposed project is an autonomous quadrotor navigation system designed to move from a defined start position to a destination without using GPS as its core navigation source. The drone uses onboard inertial and complementary sensing, ROS 2-based software, a flight controller, local path planning and obstacle detection to perform autonomous navigation in a controlled environment.

## 1.2 Background

Small UAVs are increasingly used for inspection, search and rescue, monitoring, mapping and other tasks where aerial mobility is useful. Autonomous navigation requires several capabilities to work together: state estimation, planning, perception, control and safety monitoring. Recent reviews emphasize that GNSS-independent UAV navigation should be treated as a complete pipeline rather than as a localization-only problem. [Chang et al., 2023; Gyagenda et al., 2022]

## 1.3 Why GPS-denied Navigation Matters

GNSS/GPS may be unavailable or degraded indoors and can also be unreliable in environments with obstructions or interference. An IMU is useful because it provides high-rate angular-velocity and acceleration measurements, but integrating those measurements causes accumulated error. Research on GPS-denied UAV navigation therefore commonly introduces complementary measurements such as vision, LiDAR, UWB, altitude or optical flow to constrain drift. [Wang et al., 2013; Gyagenda et al., 2022; Ayala et al., 2024]

## 1.4 Significance

Demonstrates an important robotics problem involving sensing, estimation, planning and control.

Provides a realistic application of ROS 2 communication and modular node design.

Allows the team to validate the architecture in simulation before attempting physical flight.

Creates measurable engineering outcomes rather than depending only on a demonstration.

Provides a foundation for future GPS-denied mapping, visual-inertial odometry or advanced obstacle avoidance.

# 2. Literature Survey

The following survey emphasizes GPS-denied/GNSS-independent UAV navigation, state estimation, obstacle avoidance, planning and autonomous-control architectures. Sources are selected from 2013–2024 because foundational papers remain directly relevant; the recent core literature is concentrated in 2021–2024.

| Source | Year | Problem / Method | Key Result | Limitation | Project Relevance |
| --- | --- | --- | --- | --- | --- |
| Y. Chang, Y. Cheng, U. Manzoor, and J. Murray, “A review of UAV autonomous navigation in GPS-denied environments,” Robotics and Autonomous Systems, vol. 170, 2023, Art. 104533. | 2023 | Survey of complete UAV autonomous-navigation strategies in GPS-denied environments. Method: Taxonomy based on map usage and navigation strategy; comparison using navigation metrics. | Highlights multiple navigation strategies and proposes evaluation measures such as path length, deviation and exploration efficiency. | Survey/review rather than a single deployable student architecture. | Directly frames the project's navigation problem and evaluation metrics. |
| N. Gyagenda, J. V. Hatilima, H. Roth, and V. Zhmud, “A review of GNSS-independent UAV navigation techniques,” Robotics and Autonomous Systems, vol. 152, 2022, Art. 104069. | 2022 | Navigation when GNSS is unavailable. Method: Bottom-up review of perception, localization and motion-planning components. | Shows that GNSS-independent navigation is a combination of multiple complementary subsystems, not localization alone. | Review paper; specific hardware trade-offs depend on implementation. | Supports separating perception, localization and planning in the system architecture. |
| W. Youn, H. Ko, H. Choi, I. Choi, J. H. Baek, and H. Myung, “Collision-free Autonomous Navigation of a Small UAV Using Low-cost Sensors in GPS-denied Environments,” International Journal of Control, Automation and Systems, vol. 19, no. 2, pp. 953–968, 2021. | 2021 | Autonomous collision-free flight of a small UAV without GPS. Method: Low-cost onboard sensing, error-state Kalman filtering, visual odometry, altitude measurement, SLAM and collision-free planning. | Demonstrated autonomous indoor flight and real-time operation on limited onboard computing. | Uses RGB-D/LiDAR/altimeter and a more complete SLAM pipeline than the proposed minimum prototype. | Strong benchmark for sensor fusion, mapping and obstacle avoidance. |
| H. Lu, H. Shen, B. Tian, X. Zhang et al., “Flight in GPS-denied environment: Autonomous navigation system for micro-aerial vehicle,” Aerospace Science and Technology, vol. 124, 2022, Art. 107521. | 2022 | Fast and robust MAV navigation in GPS-denied environments with limited onboard resources. Method: Integrated platform, perception, motion planning and control architecture. | Experimental testing reported fast and robust autonomous navigation. | Research-grade platform and architecture; not a direct low-cost student build recipe. | Supports the end-to-end architecture and resource-constrained design. |
| A. Ayala, L. Portela, F. Buarque, B. J. T. Fernandes, and F. Cruz, “UAV control in autonomous object-goal navigation: a systematic literature review,” Artificial Intelligence Review, vol. 57, no. 5, Art. 125, 2024. | 2024 | Understanding the components and open challenges of autonomous UAV object-goal navigation. Method: Systematic review of 67 articles from major scientific databases. | Identifies dependencies among control, planning, localization and mapping, and highlights standardization and comparison challenges. | Review of literature rather than an experimental platform. | Supports the project's modular decomposition and evaluation plan. |
| H.-Y. Lin and J.-R. Zhan, “GNSS-denied UAV indoor navigation with UWB incorporated visual inertial odometry,” Measurement, vol. 201, 2022, Art. 112256. | 2022 | Improving UAV localization accuracy in indoor GNSS-denied environments. Method: Combines visual-inertial odometry with UWB global constraints using nonlinear optimization. | Reports smoother localization trajectories in a global coordinate frame. | Requires UWB and visual sensing, increasing system complexity. | Demonstrates the value of complementary measurements to reduce inertial/visual drift. |
| F. J. Perez-Grau, R. Ragel, F. Caballero, A. Viguria, and A. Ollero, “An architecture for robust UAV navigation in GPS-denied areas,” Journal of Field Robotics, vol. 35, pp. 121–145, 2018. | 2018 | Safe and reliable autonomous UAV navigation without GPS. Method: Software architecture combining localization and motion planning, including visual odometry and a Lazy Theta* planning variant. | Presents an integrated architecture aimed at robust autonomous navigation. | Older than the project's preferred 2019–2026 literature window. | Useful foundational architecture reference and shows that robust navigation requires multiple modules. |
| T. Wang, C. Wang, J. Liang, Y. Chen, Y. Zhang et al., “Vision-Aided Inertial Navigation for Small Unmanned Aerial Vehicles in GPS-Denied Environments,” 2013. | 2013 | Inertial navigation drift for small UAVs without GPS. Method: Visual motion estimation fused with IMU through an indirect EKF. | Reported reduced computational complexity with similar estimate accuracy to an augmented filter in the described setup. | Older work and vision-dependent. | Clearly explains why IMU-only navigation drifts and why aiding measurements are needed. |
| G. Hemann, S. Singh, and M. Kaess, “Long-range GPS-denied Aerial Inertial Navigation with LIDAR Localization,” Proceedings of IEEE/RSJ IROS, pp. 1659–1666, 2016. | 2016 | Long-range aerial inertial navigation without GPS. Method: Error-state Kalman filtering with IMU bias estimation and intermittent LiDAR localization against a terrain model. | Demonstrated 218 km flight data with 27 m final position error in the reported experiments. | Full-scale aircraft and terrain-map matching are far beyond undergraduate scope. | Strong evidence that inertial drift needs periodic external localization. |
| M. A. K. et al., “Autonomous Unmanned Aerial Vehicle navigation using Reinforcement Learning: A systematic review,” Engineering Applications of Artificial Intelligence, vol. 115, 2022, Art. 105321. | 2022 | Autonomous UAV navigation and obstacle avoidance using reinforcement learning. Method: Systematic review of reinforcement-learning approaches for UAV navigation. | Highlights the growing use of RL and the importance of algorithm/environment selection. | RL introduces training, safety and generalization complexity that is unnecessary for the minimum prototype. | Supports leaving learning-based navigation as future scope rather than core semester scope. |

## 2.1 Literature Comparison and Findings

GNSS-independent navigation is consistently presented as a multi-component problem involving perception, localization/state estimation and motion planning.

IMU measurements are valuable for high-rate motion estimation but cannot by themselves provide bounded long-term position accuracy because integration error accumulates.

Complementary measurements such as vision, LiDAR, UWB, altitude and optical flow can constrain drift or improve observability.

Obstacle avoidance requires a separate perception/safety layer that can modify or interrupt planned motion.

Research-grade systems often combine several sensing modalities and SLAM; this motivates a smaller, controlled minimum viable prototype for the semester.

Learning-based navigation is an active research direction, but it adds training and validation complexity and is therefore better treated as future scope for this project.

# 3. Problem Statement & Research/Engineering Gap

## 3.1 Formal Problem Statement

Conventional small-drone autonomy often depends on GNSS/GPS for global position. In indoor, urban-obstructed, underground, or otherwise GNSS-denied environments, that position source can be unavailable or unreliable. An IMU provides high-rate motion information but its integrated position estimate accumulates drift. Therefore, a practical autonomous drone needs an onboard state-estimation pipeline that combines IMU data with one or more complementary measurements, together with local obstacle sensing and a navigation/controller stack. This project addresses the engineering problem of building and validating such an integrated pipeline on a student-scale drone in a controlled environment.

## 3.2 Existing Limitations

Manual control requires continuous pilot attention and does not provide repeatable autonomous trajectories.

GPS-dependent navigation is unsuitable as the primary localization source in environments where GNSS is unavailable or unreliable.

IMU-only position estimation accumulates drift over time.

Obstacle detection and avoidance cannot be assumed to follow automatically from localization; they require explicit sensing and decision logic.

High-end research systems can require cameras, LiDAR, UWB, motion-capture systems or powerful onboard computers, increasing cost and integration complexity.

## 3.3 Engineering Gap Addressed

The proposed contribution is not a claim that GPS-denied navigation, ROS 2, IMUs or obstacle avoidance are individually novel. Instead, the engineering gap is the development and validation of a compact, modular, student-scale integration in which onboard sensing, local state estimation, goal-directed navigation, obstacle safety logic and PX4/ROS 2 flight control operate as one measurable system under constrained hardware and controlled test conditions.

## 3.4 Defensible Gaps

Practical integration gap: research often describes advanced multi-sensor systems; a semester prototype needs a reduced architecture with clearly defined interfaces and testable behavior.

Drift-management gap: an IMU-only approach is insufficient for sustained position navigation, motivating a controlled evaluation of complementary aiding such as optical flow and range.

Safety-integration gap: navigation success should include obstacle-response and recovery behavior, not only position estimation accuracy.

Evaluation gap: the project can contribute a repeatable test protocol measuring success rate, goal error, path length, obstacle response time and recovery success.

Resource gap: the system should be designed around accessible student hardware and simulation-first validation rather than research-grade infrastructure.

## 3.5 What Should NOT Be Claimed as Novel

Using ROS 2.

Using PX4.

Using an IMU.

Using optical flow or LiDAR.

Using a Kalman/EKF-style estimator.

Using obstacle avoidance.

Using Gazebo.

Calling a drone autonomous simply because it follows waypoints.

# 4. Aim, Objectives & Requirements

## 4.1 Main Aim

To design and develop a ROS 2-based autonomous drone navigation system that can travel from a defined start position toward a destination, estimate its motion without GPS, detect nearby obstacles, and generate safe navigation commands in a controlled environment.

## 4.2 Specific Objectives

Integrate a flight controller with a ROS 2 companion-computer stack.

Acquire and process IMU data and complementary aiding measurements for local state estimation.

Estimate the drone attitude and local motion without using GPS as the navigation source.

Generate a route or waypoint trajectory from a start position to a destination.

Detect obstacles within the drone's local flight corridor using onboard range sensing.

Modify or stop the planned motion when an obstacle creates an unsafe path.

Evaluate navigation accuracy, obstacle avoidance, completion rate, and response time in simulation and controlled tests.

## 4.3 Functional Requirements

Arm/takeoff and controlled flight through the flight controller.

Read IMU and range/altitude sensing data.

Estimate local pose/velocity.

Accept a destination or waypoint goal.

Generate and update a safe local trajectory.

Detect an obstacle inside a configurable safety zone.

Stop, hover, reroute, or return to a safe state after a navigation fault.

Log navigation and sensor data for evaluation.

## 4.4 Non-functional Requirements

Modular ROS 2 architecture.

Deterministic and inspectable data flow.

Low-latency obstacle response.

Recoverable failure behavior.

Simulation-first validation.

Safe operation in a controlled indoor test area.

No dependence on GPS for the core navigation demonstration.

## 4.5 Project Boundaries

Core semester scope is local autonomous navigation in a controlled indoor or simulated environment.

GPS/GNSS may be present for bench comparison or safety telemetry, but is not used as the core navigation source.

The baseline system does not require a camera or deep-learning perception stack.

The baseline obstacle sensor should be a simple range sensor such as ToF/LiDAR; ultrasonic can be used for low-cost experiments where its limitations are acceptable.

Full outdoor long-range GPS-denied navigation, dense 3-D SLAM, dynamic-obstacle prediction, and reinforcement learning are future scope.

Physical flight is performed only after simulation, bench tests, tethered/low-risk tests, and safety checks.

# 5. Methodology & System Architecture

## 5.1 Proposed Methodology

1. Pre-flight checks and sensor calibration.

2. Initialize PX4/flight controller and ROS 2 nodes.

3. Establish a local reference frame and validate state-estimator health.

4. Arm and take off to a safe test altitude.

5. Receive the destination/waypoint goal.

6. Estimate local pose and velocity continuously from onboard sensing.

7. Generate a route toward the destination.

8. Monitor the obstacle safety zone continuously.

9. If the path is clear, send trajectory/position setpoints to the flight controller.

10. If an obstacle is detected, slow/stop/hover and generate an alternative local trajectory.

11. Continue until the goal tolerance is reached.

12. Land and log the mission result.

13. If a critical fault occurs, enter the predefined safe state rather than continuing autonomous motion.

## 5.2 High-Level Architecture

Sensors → State Estimation → ROS 2 Navigation → Path Planning → Obstacle Avoidance → PX4 Flight Controller → ESCs/Motors → Drone

The architecture separates high-level autonomy from low-level stabilization. ROS 2 handles mission logic, state-estimation integration, planning, obstacle monitoring and setpoint generation. PX4 remains responsible for flight-control functions and actuator-level stabilization. PX4 documents ROS 2 integration through its uXRCE-DDS middleware, with a client on the flight-controller side and an agent on the companion computer. [PX4 ROS 2 documentation]

## 5.3 ROS 2 Node Architecture

| Node | Responsibility |
| --- | --- |
| imu_sensor / flight_controller | Publishes IMU, attitude and flight-state data. |
| range_sensor_node | Publishes altitude and obstacle-distance measurements. |
| state_estimator | Fuses IMU with aiding measurements and produces local pose/velocity. |
| obstacle_monitor | Converts range measurements into safety constraints/obstacle states. |
| mission_manager | Stores mission state, destination and recovery logic. |
| global_or_waypoint_planner | Creates a coarse route/waypoint sequence. |
| local_planner | Generates a collision-aware local trajectory. |
| trajectory_controller | Converts the selected trajectory into flight setpoints. |
| px4_bridge | Exchanges ROS 2 data with PX4 through the supported PX4 ROS 2 middleware. |
| logger / rviz / diagnostics | Visualizes and records state, paths, obstacles and failures. |

## 5.4 ROS 2 Interfaces

ROS 2 topics are appropriate for continuous sensor/state streams, services for short request/response operations, and actions for longer-running behaviors that require feedback or cancellation. [ROS 2 documentation]

| Interface | Type | Purpose |
| --- | --- | --- |
| /sensors/imu | sensor_msgs/Imu | Continuous IMU measurements |
| /sensors/range | sensor_msgs/Range or sensor-specific message | Distance/altitude sensing |
| /state/odom | nav_msgs/Odometry | Estimated local pose and velocity |
| /navigation/goal | geometry_msgs/PoseStamped | Current destination |
| /navigation/path | nav_msgs/Path | Planned route |
| /navigation/local_path | nav_msgs/Path | Local collision-aware trajectory |
| /navigation/obstacle_state | custom message / standard diagnostic | Obstacle state and minimum distance |
| /flight/setpoint | PX4 message or project interface | Trajectory/position setpoint |
| /flight/state | PX4 message or project interface | Flight mode/arming/status |

Suggested services/actions:

| Type | Name | Purpose |
| --- | --- | --- |
| Service | /navigation/set_goal | Set or replace a short-lived goal request. |
| Service | /navigation/reset_estimator | Reset local-estimation origin after a controlled restart. |
| Service | /navigation/get_status | Return current mission and safety state. |
| Action | /navigation/navigate_to | Long-running goal with feedback, cancellation and result. |
| Action | /navigation/return_to_safe_point | Long-running recovery maneuver. |

## 5.5 TF2 Frame Tree

map/local_origin → odom → base_link → sensor frames
                         ├→ imu_link
                         ├→ range_front_link
                         ├→ range_down_link
                         └→ optical_flow_link (optional)

## 5.6 State Estimation

The IMU supplies high-rate angular velocity and linear acceleration. A state estimator propagates the vehicle state between aiding measurements. To limit drift, the project should use a complementary measurement such as optical flow, downward range/altitude or another local position/velocity source. The exact estimator can be an EKF/error-state EKF or an existing flight-stack estimator, provided the team clearly identifies what data are fused and what output is consumed by the navigation layer. The literature supports this design principle: vision/IMU fusion, ESKF with visual and altitude measurements, UWB-aided VIO and LiDAR-assisted inertial navigation have all been used to constrain GPS-denied drift. [Wang et al., 2013; Youn et al., 2021; Lin & Zhan, 2022; Hemann et al., 2016]

## 5.7 Path Planning and Obstacle Avoidance

The first implementation should use a simple waypoint or grid-based route in a known/controlled environment. A local safety layer continuously checks range measurements. If the forward path is blocked, the local planner can generate a detour around a predefined obstacle region or command a safe hover/stop while a new route is computed. This is intentionally simpler than full 3-D dynamic obstacle prediction. Reviews show that UAV path planning and obstacle avoidance remain active research areas with trade-offs among computational cost, path optimality, energy and safety. [Chen et al., 2026 review]

## 5.8 Gazebo/RViz2 Role

Simulation is used to build the ROS 2 graph, validate sensor topics, test navigation logic, inject obstacles and faults, and record repeatable metrics before physical flight. Gazebo supports ROS 2 integration and can exchange data/commands with ROS 2; RViz can be used for visualization of state, TF and navigation information. [Gazebo documentation]

# 6. Hardware Feasibility & Cost

| Component | Purpose | Estimated INR range |
| --- | --- | --- |
| Quadrotor frame + motors + ESCs | Provides the airframe and propulsion. | ₹8,000–₹18,000 |
| PX4-compatible flight controller | Low-level stabilization, sensor handling and actuator control. | ₹12,000–₹40,000 |
| Companion computer | Runs ROS 2, state estimation, planning and mission logic. | ₹6,000–₹12,000 |
| IMU | High-rate angular velocity and acceleration measurements; often integrated in flight controller. | ₹0–₹4,000 incremental |
| Optical-flow sensor (optional but recommended) | Provides planar motion aiding relative to the ground. | ₹3,000–₹16,000 |
| Range sensor / ToF / LiDAR | Altitude and obstacle-distance measurements. | ₹1,000–₹15,000+ |
| Battery + power distribution + regulator | Supplies motors and electronics. | ₹3,000–₹7,000 |
| Propellers, wiring, mounts, spares | Integration and maintenance items. | ₹2,000–₹5,000 |

Estimated student-project ranges; actual prices vary by supplier, specification and availability. For example, current Indian listings show a Pixhawk 6C plastic around ₹22,499 and an aluminium version around ₹34,999, while simple HC-SR04 ultrasonic modules are around ₹70–₹90. An Indian optical-flow module listing is around ₹2,999, while a combined optical-flow/range/IMU module is listed around ₹15,000. These are reference listings, not mandatory purchases.

A practical minimum prototype can therefore be built around an existing or student-grade quadrotor, a PX4-compatible flight controller, a Raspberry Pi-class companion computer, IMU data already available from the flight controller, one optical-flow or local-motion aid, and a range sensor for obstacle detection. The exact selection should be finalized after checking payload, voltage, interface and flight-controller compatibility.

## 6.1 Main Integration Risks

Sensor vibration and poor mounting can degrade IMU/optical-flow measurements.

Frame transformations and sensor alignment errors can corrupt navigation.

An IMU-only estimator will drift rapidly in position.

Wireless communication should not be treated as the sole safety mechanism.

Offboard control must respect the flight controller's mode, arming and setpoint requirements.

Range sensors may fail on soft, angled, transparent or acoustically difficult surfaces.

Physical-flight testing introduces safety risks that are absent in simulation.

## 6.2 Calibration Requirements

IMU bias and scale calibration.

Sensor-to-body extrinsic orientation measurement.

Range-sensor mounting and distance validation.

Optical-flow ground-height and lighting checks if used.

Motor/ESC and propeller inspection and balancing.

Flight-controller attitude and level calibration.

Coordinate-frame/sign-convention verification.

# 7. Evaluation Plan

| Metric | Definition | Suggested measurement method |
| --- | --- | --- |
| Navigation success rate | Percentage of trials reaching the goal without manual takeover. | Record ROS 2/PX4 logs over repeated trials and compute the metric. |
| Position/goal error | Distance between final estimated position and the target position. | Record ROS 2/PX4 logs over repeated trials and compute the metric. |
| Trajectory/path length | Distance travelled compared with the planned route. | Record ROS 2/PX4 logs over repeated trials and compute the metric. |
| Obstacle response time | Time from obstacle detection to safe avoidance/stop command. | Record ROS 2/PX4 logs over repeated trials and compute the metric. |
| Collision/near-collision rate | Number of trials with contact or breach of the safety distance. | Record ROS 2/PX4 logs over repeated trials and compute the metric. |
| Waypoint completion rate | Percentage of commanded waypoints reached within tolerance. | Record ROS 2/PX4 logs over repeated trials and compute the metric. |
| Recovery success rate | Percentage of injected faults from which the system returns to a safe state. | Record ROS 2/PX4 logs over repeated trials and compute the metric. |
| Compute/communication latency | Measured processing and message delay for critical navigation paths. | Record ROS 2/PX4 logs over repeated trials and compute the metric. |

## 7.1 Test Scenarios

Straight-line navigation with no obstacles.

Navigation through two or more waypoints.

Single static obstacle placed in the nominal path.

Multiple static obstacles requiring a detour.

Temporary obstacle introduced during flight in simulation.

Sensor dropout or invalid range measurement.

Estimator reset/reinitialization test.

Communication interruption or stale command test.

Goal reached within the defined tolerance.

Abort/recovery test leading to a safe hover or landing state.

## 7.2 Proposed Engineering Targets

The following are proposed targets for validation, not experimental results: achieve at least 80% successful autonomous goal completion in the controlled test set; maintain a configurable safety margin from obstacles; detect and respond to a simulated obstacle within 0.5–1.0 s; complete at least 90% of nominal waypoints within the chosen position tolerance; and recover safely from at least 80% of deliberately injected software/sensor faults. The team should revise these numbers after initial simulation baselines rather than claiming them as achieved.

# 8. Research/Engineering Contribution

A conservative contribution statement is: “We develop and experimentally evaluate a modular ROS 2 autonomous-drone navigation prototype that combines inertial sensing, complementary local sensing, goal-directed navigation, obstacle safety logic and PX4 flight control in a controlled GPS-independent environment.”

A balanced contribution statement is: “The project investigates a resource-conscious architecture for GPS-denied local navigation and evaluates how complementary sensing and obstacle-aware planning affect navigation success, position error and recovery behavior on a student-scale platform.”

An ambitious contribution should only be used if supported by experiments: “The project demonstrates a robust autonomous navigation architecture capable of sustained GPS-denied flight across varied environments with quantified improvements over an IMU-only baseline.”

# 9. Final DA-1 Report Structure

Title Page

Project Idea, Background and Significance

Literature Survey

Problem Statement

Research/Engineering Gap

Aim and Objectives

Functional and Non-functional Requirements

Project Scope and Boundaries

Methodology

Hardware Architecture

ROS 2 Software Architecture

State Estimation

Path Planning and Obstacle Avoidance

Simulation and Testing Plan

Evaluation Metrics

Expected Outcomes

Future Scope

References

# 10. 12-Slide Presentation Content

## 1. Title

Autonomous Drone Navigation
GPS-independent local navigation with ROS 2, onboard sensing and obstacle avoidance.

## 2. Background & Significance

UAV autonomy requires sensing, localization, planning and control.
GPS/GNSS may be unavailable indoors or degraded in constrained environments.
Project focus: controlled GPS-independent navigation.

## 3. Problem Statement

Manual control is not fully autonomous.
GPS dependence limits operation in GNSS-denied areas.
IMU-only position estimates drift.
Obstacle avoidance requires a dedicated sensing and safety layer.

## 4. Existing Approaches

IMU + GPS/INS
Visual-inertial odometry
LiDAR/IMU localization
UWB-aided localization
Map-based and mapless navigation
Learning-based navigation

## 5. Literature Survey

Show 6–8 key papers in a compact comparison table.
Highlight: sensor fusion, GPS-denied localization, obstacle avoidance, planning and evaluation.

## 6. Research/Engineering Gap

Need a compact student-scale integration.
Separate estimation, planning, obstacle safety and flight control.
Evaluate the complete pipeline using repeatable metrics.

## 7. Objectives

Integrate ROS 2 and PX4.
Estimate local motion without GPS.
Navigate to a goal.
Detect obstacles.
Avoid/stop safely.
Measure performance.

## 8. Methodology

Sensors → State estimation → Goal/waypoint planner → Local obstacle monitor → Trajectory/setpoint generation → PX4 → Motors.

## 9. Hardware Architecture

Quadrotor + motors/ESCs
PX4-compatible flight controller
Raspberry Pi-class companion computer
IMU
Optical flow/local-motion aid
Range sensor
Battery/power system

## 10. ROS 2 Architecture

Sensor nodes
State estimator
Mission manager
Global/waypoint planner
Local planner
Obstacle monitor
PX4 bridge
Logger/RViz2

## 11. Evaluation

Navigation success rate
Goal error
Path length
Obstacle response time
Collision/near-collision rate
Waypoint completion
Recovery success
Latency

## 12. Future Scope & References

Visual-inertial odometry
3-D SLAM
Dynamic obstacle prediction
UWB/LiDAR fusion
Learning-based navigation
Larger outdoor experiments
Include verified literature and official ROS 2/PX4 references.

# 11. Conclusion

The proposed Autonomous Drone Navigation project is technically meaningful but must be scoped carefully. The central lesson from the literature is that GPS-denied navigation is not solved by an IMU alone: inertial data provide high-rate motion information, while complementary measurements are needed to constrain drift. The recommended semester architecture therefore combines a PX4 flight controller, ROS 2 companion-computer software, IMU-based state estimation, a complementary local-motion/altitude measurement, range-based obstacle monitoring and a simple goal-directed planner. Simulation-first development makes the project substantially more achievable and provides repeatable evaluation before physical flight.

# 12. References

[1] Y. Chang, Y. Cheng, U. Manzoor, and J. Murray, “A review of UAV autonomous navigation in GPS-denied environments,” Robotics and Autonomous Systems, vol. 170, 2023, Art. 104533.
https://doi.org/10.1016/j.robot.2023.104533

[2] N. Gyagenda, J. V. Hatilima, H. Roth, and V. Zhmud, “A review of GNSS-independent UAV navigation techniques,” Robotics and Autonomous Systems, vol. 152, 2022, Art. 104069.
https://doi.org/10.1016/j.robot.2022.104069

[3] W. Youn, H. Ko, H. Choi, I. Choi, J. H. Baek, and H. Myung, “Collision-free Autonomous Navigation of a Small UAV Using Low-cost Sensors in GPS-denied Environments,” International Journal of Control, Automation and Systems, vol. 19, no. 2, pp. 953–968, 2021.
https://doi.org/10.1007/s12555-019-0797-7

[4] H. Lu, H. Shen, B. Tian, X. Zhang et al., “Flight in GPS-denied environment: Autonomous navigation system for micro-aerial vehicle,” Aerospace Science and Technology, vol. 124, 2022, Art. 107521.
https://doi.org/10.1016/j.ast.2022.107521

[5] A. Ayala, L. Portela, F. Buarque, B. J. T. Fernandes, and F. Cruz, “UAV control in autonomous object-goal navigation: a systematic literature review,” Artificial Intelligence Review, vol. 57, no. 5, Art. 125, 2024.
https://doi.org/10.1007/s10462-024-10758-7

[6] H.-Y. Lin and J.-R. Zhan, “GNSS-denied UAV indoor navigation with UWB incorporated visual inertial odometry,” Measurement, vol. 201, 2022, Art. 112256.
https://doi.org/10.1016/j.measurement.2022.112256

[7] F. J. Perez-Grau, R. Ragel, F. Caballero, A. Viguria, and A. Ollero, “An architecture for robust UAV navigation in GPS-denied areas,” Journal of Field Robotics, vol. 35, pp. 121–145, 2018.
https://doi.org/10.1002/rob.21757

[8] T. Wang, C. Wang, J. Liang, Y. Chen, Y. Zhang et al., “Vision-Aided Inertial Navigation for Small Unmanned Aerial Vehicles in GPS-Denied Environments,” 2013.
https://doi.org/10.5772/56660

[9] G. Hemann, S. Singh, and M. Kaess, “Long-range GPS-denied Aerial Inertial Navigation with LIDAR Localization,” Proceedings of IEEE/RSJ IROS, pp. 1659–1666, 2016.
https://publications.ri.cmu.edu/long-range-gps-denied-aerial-inertial-navigation-with-lidar-localization

[10] M. A. K. et al., “Autonomous Unmanned Aerial Vehicle navigation using Reinforcement Learning: A systematic review,” Engineering Applications of Artificial Intelligence, vol. 115, 2022, Art. 105321.
https://doi.org/10.1016/j.engappai.2022.105321

[11] ROS 2 Documentation — Interfaces: Topics, Services and Actions. https://docs.ros.org/en/humble/Concepts/Basic/Interfaces-Topics-Services-Actions.html

[12] ROS 2 Documentation — Understanding Nodes. https://docs.ros.org/en/humble/Tutorials/Beginner-CLI-Tools/Understanding-ROS2-Nodes/Understanding-ROS2-Nodes.html

[13] PX4 Guide — uXRCE-DDS (PX4-ROS 2/DDS Bridge). https://docs.px4.io/main/en/middleware/uxrce_dds

[14] PX4 Guide — ROS 2 User Guide. https://docs.px4.io/main/en/ros/ros2_comm

[15] Gazebo Documentation — ROS 2 Integration / Simulation Interfaces. https://gazebosim.org/docs/latest/ros2_sim_interfaces/